|  | Risk Assessment | Page 1 of 3 Issue v3 Date form issued: 09/08/11 Revised: 09/08/11 Form No:HST/22 |
| --- | --- | --- |

| Task/Activity: SS07 – Sign Shop Activities (outside the depot) | Date assessment completed: 30-9-25 | Review Date: 30-9-25 |
| --- | --- | --- |
| Brief Details of Task/Activity Undertaking routine sign maintenance | # Assessment completed by: Stewart Newsham | Signature: |

| What are the hazards? e.g. slip/trip hazards, electricity, manual handling, work equipment | ## Who might be harmed and how? ## e.g. staff, service users, visitors etc... and likely injury e.g. bruises, muscle strain, fracture, poisoning etc… | What are you already doing to control the hazard? | What further action or additional controls are required (if necessary) | Risk rating (after control measures) | Action by who | Action by when | Date completed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vibration Use of powered hand tools Use of construction equipment and plant Over exposure to vibration, e.g. ensure rest breaks are taken and do not exceed maximum operating times as tagged on machinery. | Operatives Whole body vibration injuries (physical and neurological). Musculoskeletal problems. | Vibration levels assessed on individual items of machinery. Vary work types when possible to reduce repetitiveness. Operatives to follow correct HAVS arrangements. Operatives to wear correct PPE Monitor operative vibration exposure levels and annual assessment via OHU. Daily safety checks carried out by the user on machinery and all faults are to be reported immediately and replaced/repaired as necessary Vibration levels assessed on machinery. Vary work types when possible to reduce repetitiveness. Operatives to follow correct HAVS arrangements. Remove from service any machinery with higher vibration levels. Monitor operative vibration exposure levels and annual HAVs assessment via OHU Daily safety checks carried out by the user on machinery and all faults are to be reported immediately and replaced/repaired as necessary. Staff awareness training completed to ensure staff aware of the danger of vibration | Mandatory training for all operatives using the machinery to be reviewed if significant changes in the operation of the equipment. Ensure PPE is being worn as advised and replace as necessary. Mandatory training for all operatives using the machinery to be reviewed if significant changes in the operation of the equipment. | E |  |  |  |
| Construction/Silica dust Silica dust created by construction operations on tarmac concrete, mortar or sandstone products | Employees and the public. Respiratory illnesses and asthma, other breathing problems. | Adequate ventilation when using hazardous substances. Dust suppression equipment / water kits fitted and used at all times. All faults are to be reported immediately and machinery replaced/repaired as necessary. Limit persons near to dust creating machinery – move materials to dedicated cutting stations wherever possible. Use alternative methods of cutting is possible ie block splitter or water table saw rather than cut off saw. Limit the number of cuts required if possible Face Fit testing undertaken for all operatives. Ensuring operatives are aware that added or reduced body weight can alter the shape of the face and fit of the mask. Beards or stubble will prevent a full seal of the fitted mask and alternatives / re fitting will be required Operatives to have personal first aid kit on site vehicle which is up to date and replenished as necessary. Suitable P.P.E to be worn at all times. Vary work to avoid over exposure. Rotate operatives using machinery. Eye wash available as necessary. | Refresher training TBA by operations service for cut off saws and advise workers of the risks of RCS and ensure Face Fitting is up to date. Ensure PPE is being worn as advised and replace consumables as necessary. | E |  |  |  |
| Manual handling, bending, twisting and lifting | Employees Musculoskeletal injuries, sprains, strains. Slips,trips and falls. | Operatives to have Manual Handling training Suitable P.P.E to be worn at all times. Ensure working area safe/tidy | Operations Team to arrange Manual Handling refresher training every three years. Ensure PPE is being worn as advised and replace as necessary. | D |  |  |  |
| Noise Use of powered hand tools Use of construction equipment and plant High Levels of Noise | Operative/members of the public Loss of hearing or hearing impairment | Operatives to wear correct PPE at all times. Hearing protection issued and annual audiometry testing via OHU Where possible noise suppression devices/measures to be employed at source. Regular servicing and daily safety checks carried out by the user on machinery and all faults are to be reported immediately and equipment is to be replaced/repaired as necessary. | Maintain operatives/supervisor training. Ensure PPE is being worn as advised and replace as necessary, choice of external defenders or internal plugs. Advise supervisor / seek medical advice as soon as hearing loss is suspected. | E |  |  |  |
| Using a straight sided cutting saw (stihl type) Cutting of paving/surfacing materials Over exposure to vibration Fast moving mechanical parts Noise Silica/dust | Operatives / members of the public Whole body vibration injuries (physical and neurological). Musculoskeletal problems. Hearing loss/impairment Eye injury. Dust inhalation, respiratory illness | Vibration levels assessed on individual items of machinery. (SEE HAVS VALUES LIST) Vary work types when possible to reduce repetitiveness. Operatives to follow correct HAVS arrangements. Operatives to wear correct PPE Monitor operative vibration exposure levels and annual assessment via OHU Daily safety checks carried out by the user on machinery and all faults are to be reported immediately and replaced/repaired as necessary All users to be trained and certificated in the use of diamond blades and straight sided cut off saws. Daily safety checks to ensure saw and blade are in good condition. Saw to be replaced if found to be in poor condition / vibrating or smoking excessively Safe working area set up and correctly signed, coned and barriered in accordance with Chapter 8 of the Traffic Signs Manual Employees to wear hi vis clothing and PPE Operatives to have personal first aid kit on site which is up to date and replenished as necessary. Eyewash available on all sites replenished as necessary. Ear defenders/plugs to be worn during use Monitor exposure levels and annual audiometry screening for all operatives via OHU | Ops team to maintain operative/supervisor training. Check PPE is being worn as advised operative to replace as necessary. Mandatory training for all operatives using the machinery, to be reviewed if significant changes in the operation of the equipment. Ensure PPE is being worn as advised and replace as necessary All cutting to be wet cutting in a safe location as far from the public as is practicable. Operatives to have personal first aid kit on site which is up to date and replenished as necessary. Eyewash available on all sites replenished as necessary. Other employees instructed to keep clear. Appropriate PPE provided and used. Face fit testing undertaken at the prescribed intervals for all employees | E |  |  |  |
| Hand Excavation Use of pneumatic tools and pick/spade/bars implements Manual handling, bending, twisting and lifting. Gas, electricity, water. Weather and environmental conditions Engine heat, noise, fumes. Moving parts, vibration of equipment. | Operatives/Members of the public Other road users. Fatality, major/minor injuries electrocution, shock, burns, cuts, bruises Climate induced injuries,from falls, slips and trips (ground conditions.) Cuts, Bruises, burns, trapped/crushed limbs, breathing problems. Hearing loss/impairment. Hand arm vibration. (H.A.Vs) | Traffic management controls to Chapter 8 of the Traffic Signs Manual. Correct PPE to be worn Regular servicing and daily checks prior to use to ensure machinery is operating correctly. Operators to have a safe working area, and correct training. Operatives to have Manual Handling training organised by ops team Operatives experienced in work activity. Service drawings supplied for all new digs and checked before commencing work. Operatives trained in the use of cable detection and avoidance equipment. Operatives to wear correct PPE at all times. Annual calibration of cable avoidance equipment organised by ops team Only trained and competent operators to carry out activity. | All faults are to be reported immediately and machinery replaced/repaired as necessary. | E |  |  |  |
| Hand Lay construction materials Patch tarmac/bitmac Block paving or flag reinstatement Manual handling, bending, twisting and lifting. | Operatives/members of the public Working near live traffic. / R.T.C with other road users. | Traffic management controls to Chapter 8 of the Traffic Signs Manual. Operatives to have Manual Handling training (SEE MANUAL HANDLING GUIDANCE LIST 1 | Ensure PPE is being worn as advised and replace as necessary. | E |  |  |  |
| Erection of sign plates Fall from heights Falling materials or tools Bracket fixings to poles and signs | Employees Fatality Pulled Muscles Sprains & Strains Broken Bones Cuts Traps Bruising Employees and Members of the Public Fatality Broken Bones Other Injuries Employees and public. Tools falling out of hands or off vehicle/ ladders | PPE: - Hard Hats (BS EN 397)Gloves and Footwear (BS EN345) Training awareness MEWP Training Use of trestle type self supporting aluminium ladders (BS 131 2 man operation when Necessary. Work off flat platform /vehicle with safety rails where practical. PPE: -Hard Hats (BS EN 397) Chapter 8 Regulations closing the working area/ensure observe surroundings Introduction of waist belts to hold tools if required PPE: - Gloves and Footwear (BS EN345) Training awareness | Use of MEWP when necessary | E |  |  |  |
| The use of hydraulic platform to carry out street lighting associated tasks (Banner installs) Overhead obstructions Use of tools and materials in bucket | Operatives/member of the public Fatality, Electric Shock, Burns, Broken Bones Employees & Members of the Public | PPE Gear: - Safety Harness (BS EN 361) Hard Hats (BS EN 397) MEWP Traing certificate renewable after 5 years G39 Training Tools and materials to be tethered where practicable by lanyyards to reduce the possibility of materials falling |  | E |  |  |  |
| Use of ladders | Operatives/member of the public |  |  |  |  |  |  |
| Hazardous materials | Operatives/member of the public |  |  |  |  |  |  |
| Vehicular Traffic | Operatives/member of the public |  |  |  |  |  |  |
| Weather | Operatives/member of the public |  |  |  |  |  |  |
| Violence and aggression from the public |  |  |  |  |  |  |  |
| Risk to public/pedestrians |  |  |  |  |  |  |  |

| CATEGORIES OF LIKELIHOOD |  |
| --- | --- |
| Highly Likely | Expected to happen/reoccur, possibly frequently. |
| Possible | Might happen/reoccur at some time depends on circumstances. |
| Unlikely | Not expected to happen/reoccur but possible in certain circumstances. |
| Very Unlikely | Would only occur in very exceptional circumstances. |

| CATEGORIES OF CONSEQUENCE SEVERITY |  |
| --- | --- |
| Catastrophic | Incident could result in one or more fatalities. |
| Major | Major injury resulting in incapacity, hospitalisation >24 hours. |
| Significant | Injury requires attention of a Doctor or Hospital treatment or hospitalisation <24 hours. |
| Minor | Small cut, bruise, abrasion, basic first aid treatment provided. |
| Negligible | Some discomfort, self help. No treatment required. |

| RISK RATING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Highly Likely | Possible | Unlikely | Very Unlikely |
| Catastrophic | A | A | B | E |
| Major | A | B | C | E |
| Significant | B | C | D | E |
| Minor | C | D | E | E |
| Negligible | E | E | E | E |

| RISK CLASSIFICATIONS |  |
| --- | --- |
| A | Unacceptable risk, requires immediate attention. Work should not be started or continued until the level of risk has been reduced. |
| B | High risk, requires immediate attention. Control measures must be identified and put into place as soon as possible. |
| C | Medium risk, requires attention as soon as possible. The risk should be only be tolerated in the short term and only when further control measures are being planned and introduced, Timescales must be short. |
| D | Low risks, confirm that there are no low/no cost solutions which may eliminate/ reduce the risk further. |
| E | Trivial risk, no further action required but review at regular intervals to ensure controls remain effective. |