|  | Risk Assessment | Page 1 of 3 Issue v5 Date form issued: 09/08/11 Revised: 24/05/23 |
| --- | --- | --- |

| Task/Activity: DS03 Loading / unloading materials | Date assessment completed: 14/04/2026 | Review Date: 13/04/2028 |
| --- | --- | --- |
| Brief Details of Task/Activity Loading and unloading of all depot related products | # Assessment completed by: M Treen | Signature: M. Treen |

| What are the hazards? e.g. slip/trip hazards, electricity, manual handling, work equipment | ## Who might be harmed and how? ## e.g. staff, service users, visitors etc... and likely injury e.g. bruises, muscle strain, fracture, poisoning etc… | What are you already doing to control the hazard? | What further action or additional controls are required (if necessary) | Risk rating (after control measures) | Action by who | Action by when | Date completed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RTC with other employees / contact with excavator or mobile plant. | Employees working within the loading and unloading area. Staff may suffer major injuries or fatalities as a result of being struck, crushed or run over by moving plant or vehicles. Additional risk of slips, trips and falls in active loading areas where ground conditions or plant movements reduce awareness and escape routes. . | Adequate slewing room ensured before operating excavator. Crush and slew zones clearly identified and maintained. Daily inspections of machinery before use. Reversing sounders operational at all times. Drug and alcohol policy enforced. Safe working distances maintained from plant. Hard hats and steel toe capped boots worn as a minimum. Full PPE worn during operations. |  | E |  |  |  |
| Contact with equipment and machinery. | Employees Fatality, major/minor injuries, broken limbs, cuts and bruises | PPE to be worn by employees at all times, during operations. Machinery and equipment being used are checked prior to use, complete relevant checks and procedures prior to use on site. Operatives to hold relevant licenses and have had appropriate training prior to using any machinery. Daily checks of machinery prior to use Daily site check sheets completed by supervisors. Where necessary, use a banksman. |  | E |  |  |  |
| Struck by excavator arm or bucket / load falling during loading or unloading. | Employees working near loading operations. Risk of crush injuries, fractures or fatal injuries from moving arms, buckets or falling loads during lifting, slewing or tipping activities, particularly if personnel enter exclusion zones or loads become unstable. | Traffic management and segregation measures in place. All loading and unloading operations controlled by a banksman. Non-essential personnel instructed to keep clear of loading areas. Appropriate PPE worn at all times. Dangers of working near mobile plant explained to operatives. Operatives trained and licensed as required. All machinery subject to LOLER examinations at required intervals. |  | E |  |  |  |
| Manual handling of loads | Employees may suffer muscle strains, back injuries, hernias or crushed fingers/feet when lifting, pushing or pulling loads during loading or unloading tasks. | Manual handling training provided. Mechanical aids used where possible. Team lifting encouraged for heavy or awkward loads. Staff instructed not to exceed personal capability. |  | D |  |  |  |
| Restricted visibility / reversing plant | Employees may be struck or crushed by reversing vehicles or plant due to blind spots. | Reversing alarms fitted and operational. Banksman used where visibility is restricted. Defined pedestrian routes where possible. High-visibility PPE worn. |  |  |  |  |  |

| CATEGORIES OF LIKELIHOOD |  |
| --- | --- |
| Highly Likely | Expected to happen/reoccur, possibly frequently. |
| Possible | Might happen/reoccur at some time depends on circumstances. |
| Unlikely | Not expected to happen/reoccur but possible in certain circumstances. |
| Very Unlikely | Would only occur in very exceptional circumstances. |

| CATEGORIES OF CONSEQUENCE SEVERITY |  |
| --- | --- |
| Catastrophic | Incident could result in one or more fatalities. |
| Major | Major injury resulting in incapacity, hospitalisation >24 hours. |
| Significant | Injury requires attention of a Doctor or Hospital treatment or hospitalisation <24 hours. |
| Minor | Small cut, bruise, abrasion, basic first aid treatment provided. |
| Negligible | Some discomfort, self help. No treatment required. |

| RISK RATING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Highly Likely | Possible | Unlikely | Very Unlikely |
| Catastrophic | A | A | B | E |
| Major | A | B | C | E |
| Significant | B | C | D | E |
| Minor | C | D | E | E |
| Negligible | E | E | E | E |

| RISK CLASSIFICATIONS |  |
| --- | --- |
| A | Unacceptable risk, requires immediate attention. Work should not be started or continued until the level of risk has been reduced. |
| B | High risk, requires immediate attention. Control measures must be identified and put into place as soon as possible. |
| C | Medium risk, requires attention as soon as possible. The risk should be only be tolerated in the short term and only when further control measures are being planned and introduced, Timescales must be short. |
| D | Low risks, confirm that there are no low/no cost solutions which may eliminate/ reduce the risk further. |
| E | Trivial risk, no further action required but review at regular intervals to ensure controls remain effective. |