|  | Risk Assessment | Page 1 of 4 Issue v6 Date form issued: 14/01/16 |
| --- | --- | --- |

| Task/Activity: DS05 – working from ladders | Date assessment completed: 14/04/2026 | Review Date: 13/04/2028 |
| --- | --- | --- |
| Brief Details of Task/Activity Non-routine, exceptional use of ladders within the stores area where work at height cannot be avoided. | # Assessment completed by: MARK TREEN | Signature: M. Treen |

| What are the hazards? e.g. slip/trip hazards, electricity, manual handling, work equipment | ## Who might be harmed and how? ## e.g. staff, service users, visitors etc... and likely injury e.g. bruises, muscle strain, fracture, poisoning etc… | What are you already doing to control the hazard? | What further action or additional controls are required (if necessary) | Risk rating (after control measures) | Action by who | Action by when | Date completed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Falling from height | Authorised employees using ladders within the stores area. There is a risk of falls from height resulting in bruising, sprains, fractures, head injury or spinal injury, particularly due to over-reaching, incorrect ladder positioning or failure to complete pre-use checks. | Routine work at height is avoided. Any regular or planned work at height would require alternative access equipment and a task-specific risk assessment Ensure the ladder check list is filled out / completed before use. This is to be completed 6 monthly unless ladders are not used. Always face the ladders, only descend / ascend the ladders facing the ladders. Ensure you always have proper grip of the ladders. 3 points of contact. | Ladders are used only for short-duration, light-duty tasks and are not a substitute for other access equipment. | D |  |  |  |
| Collapse due to incorrect or adverse loading or wind loading | Authorised employees. Injury may occur if ladders are overloaded, incorrectly positioned or used in unsuitable conditions, resulting in falls and serious injury. . | Do not overload ladders - check the pictogram or label on the ladder for information. Do not use in windy weather. Ladders are used only for short-duration, light-duty tasks and are not a substitute for other access equipment. | Ladders are used only for short-duration, light-duty tasks and are not a substitute for other access equipment. | D |  |  |  |
| Injury to others from falling materials | Employees, other depot users, visitors, contractors and members of the public. Slips, trips and falls. Crushing, broken bones, muscle tares, head injury. Back and spinal damage. | Corden off a safe working area – exclusion zone. Provide protection from falling objects, wear appropriate PPE. |  | D |  |  |  |

| CATEGORIES OF LIKELIHOOD |  |
| --- | --- |
| Highly Likely | Expected to happen/reoccur, possibly frequently. |
| Possible | Might happen/reoccur at some time depends on circumstances. |
| Unlikely | Not expected to happen/reoccur but possible in certain circumstances. |
| Very Unlikely | Would only occur in very exceptional circumstances. |

| CATEGORIES OF CONSEQUENCE SEVERITY |  |
| --- | --- |
| Catastrophic | Incident could result in one or more fatalities. |
| Major | Major injury resulting in incapacity, hospitalisation >24 hours. |
| Significant | Injury requires attention of a Doctor or Hospital treatment or hospitalisation <24 hours. |
| Minor | Small cut, bruise, abrasion, basic first aid treatment provided. |
| Negligible | Some discomfort, self-help. No treatment required. |

| RISK RATING |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Highly Likely | Possible | Unlikely | Very Unlikely |
| Catastrophic | A | A | B | E |
| Major | A | B | C | E |
| Significant | B | C | D | E |
| Minor | C | D | E | E |
| Negligible | E | E | E | E |

| RISK CLASSIFICATIONS |  |
| --- | --- |
| A | Unacceptable risk requires immediate attention. Work should not be started or continued until the level of risk has been reduced. |
| B | High risk requires immediate attention. Control measures must be identified and put into place as soon as possible. |
| C | Medium risk requires attention as soon as possible. The risk should be only be tolerated in the short term and only when further control measures are being planned and introduced, Timescales must be short. |
| D | Low risks, confirm that there are no low/no cost solutions which may eliminate/ reduce the risk further. |
| E | Trivial risk, no further action required but review at regular intervals to ensure controls remain effective. |